# VAX alarm integration with dry contacts

## Found in this guide:

Setup

Triple Swipe to Toggle Alarm System

Single Swipe to Disarm, Triple Swipe to Arm

VAX Systems integrates Burglar Alarm systems through dry contacts to Arm and Disarm via a single swipe or triple swipe. The Alarm System sends a signal to VAX to restrict access to card readers while armed, and only certain card holders can arm or disarm using their credentials.

The VAX system requires (1) relay from the alarm system to send status (maintained contact relay) and (1) momentary input to arm and disarm the system. Either option can be normally open/closed.

Knowledge and how to program the Alarm System is required and is not covered in this guide. This guide is specific to the VAX setup, capabilities, and an overview of how our system works.

# Setup

- Navigate to Panels > Edit Panel > IO.

- Configure an Input as External Alarm Status. Wire an input from the VAX panel to the Alarm Status relay on the Alarm System.

*Associated Door is ignored when configuring an input as External Alarm Status.

- Configure a relay as Alarm Interface. Wire the relay from the VAX panel to the Active/Deactivate input on the Alarm System.

*Associated Door is ignored when configuring a relay as Alarm Interface.

- Click Save at the bottom, then at the very top ribbon, click Update Panels.

When the "External Alarm Status" input is triggered, the panel will go into an "ARMED" state and the doors will be overridden to a "Card or PIN only" state. It will then deny users while armed, and only MASTER, SUPERVISOR, or users with DISENGAGE ALARM permissions will be allowed access.

To have a card Activate or Deactivate the alarm system, the Triple swipe function can be used. With PRS-Door-Master kits, the User Action or Reader Action can be used to have a single credentials Activate/Deactive/Toggle the Alarm System.

# Triple swipe to toggle alarm system

- Navigate to Doors > Edit Door > Reader 1.

- Enable Triple Swipe.

- From the dropdown, select Toggle Alarm Interface.

- Click Save.

- Navigate to Users > Edit User.

- Enable the Disengage Alarm and Triple Swipe privilege.

- Click Save and Update Panels.

For a credential to activate or deactivate the alarm, the user will need the "Disengage Alarm" permission. To trigger the Triple Swipe action, they will need the "Triple Swipe" permission. The user can then use their credential three times or enter their pin and hit # three times to activate or deactivate the alarm interface by sending a momentary "pulse" through the "Alarm Interface" relay.

# Single swipe to disarm, triple swipe to arm

*This is only available currently with the PRS-Door-Master controllers.

- Navigate to Doors > Edit Door > Reader 1.

- Set the reader Action to Deactivate Alarm Interface.

- Enable Triple Swipe.

- From the dropdown, select Activate Alarm Interface.

- Click Save.

- Navigate to Users > Edit User.

- Enable the Disengage Alarm and Triple Swipe privilege.

- Click Save and Update Panels.

With the "External Alarm Interface" input closed, a single swipe on the configured door will trigger the "Deactivate Alarm Interface" command and send a momentary "pulse" through the "Alarm Interface" relay, thereby disarming the system. When the "External Alarm Interface" input is open due to the alarm system now being disarmed, a single credential passing through the door will not send a momentary contact because the system is already deactivated, thereby a "deactivate command" will not be sent. When entering the credential 3 times at the configured door, the "Activate Alarm Interface" command will execute and send a momentary pulse, thereby arming the system.